Introduction

ad hoc wireless network: information sharing between mobile hosts for a particular purpose

- No need for fixed infrastructure

- Each node equipped with one or more radios

- Radios can be heterogeneous

- Each node free to move

- Paths between nodes can be multi-hop

Technical challenges for Ad hoc networks: low power, no infrastructure, mobility

“Internet of Things”

“Ubiquitous computing” – Calm technology

Graph Theory & Routing Fundamentals

Complete Graph – every pair of vertices is adjacent (connected)

Connected Graph – any two nodes are connected by a path (strongly connected – same in directed graph)

Bipartite Graph – undirected graph in which the vertices can be separated in two groups. Vertices in group 1 are only connected to vertices of group 2.

Tree

- any two vertices are connected by a unique simple path,

- Connected, but become disconnected, if any edge is removed

- Acyclic

Shortest Path spanning tree is not the same as minimum spanning tree! Spst is spanned from one node -> minimum weighted from one vertex. Mst is globally optimal.

Dijkstra Algorithm 		Bellman-Ford				Floyd-Warshall Alg

Link-state routing: each node has complete network picture (topology, link costs) by flooding the information about its neighbours  independently calculates routers (e.g. Dijkstra)

Distributed Dijkstra

- no change in the algorithm

- Each node periodically floods a control message throughout the network containing link state information

- Entire topology knowledge must be maintained at each node

Distance vector routing: each node knows the links to its neighbours (no flooding). Nodes jointly calculate routes by iteratively exchanging with neighbours best-known routes to all nodes.

Distributed Bellman-Ford

Distance Vector Protocols:

- Each node maintains routing table with {destination, nex hop, distance(cost)}

- Nodes exchange routing table information with neighbours periodically or whenever table changes

- Node updates its routing table if it finds a “better” route

- Entries in the routing table are deleted if they are too old

Split Horizon with poisoned reverse:

But doesn’t work for loops with more than two nodes

Stable marriage Problem: unstable, if there is a pair who likes each other more than their spouses (they can improve their situation by dumping spouses and eloping)

Gale-Shapley Algorithm: a stable marriage always possible, found in O(n²) time

- Each man and woman list their preferences

- All people begin unengagend

- While there are unengaged men, each proposes until a woman accepts

- Unengaged women accept first proposal they get

- If an engaged woman receives a proposal she likes better, she breaks old engagement and accepts new proposal; dumped man begins proposing where he left off

Men are better off if they propose! Order of proposal is irrelevant.

Routing for Ad Hoc Networks

New routing protocols, because of

- Mobility = highly dynamic network topology

- No infrastructure for central administration

- Unreliable, bandwith-constrained wireless links

- Resource (energy)-constrained nodes

- Unreliable nodes

- Finding nodes

Key problem: every node is potentially also acting as a router

Mobile Ad Hoc Networks (MANET):

- Dynamic toplogy (nodes can join and leave the network & change the range of their transmissions)

- Each node acts as independent router

- Due to wireless communication:

  - Bandwidth-constrained and variable capacity links

  - Limited transmitter range

  - Energy-constrained

  - Limited physical security

- Distributed MAC and network protocols

- Complex routing protocols with large transmission overheads and large processing loads on each node

PROACTIVE protocols: determine routes independent of traffic pattern (low latency, high overhead)

REACTIVE protocols: determine & maintain routes only when and if needed  (high latency, low O/H)

Destination Sequenced Distance Vector Routing Protocol (DSDV): (proactive)

- Table updates with increasing sequence numer tags  prevents routing loops

- Nodes communicate their routing table periodically & when significant new information available

- Broken link is given a metric of ∞ and an update sequence number  immediately broadcast to all neighbours

- Incremental updates possible – only info which changed since the last update

- A node comparing two routes, take the one with a higher sequence number

+	existing wired network protocol can be applied to MANET with minimal modification (sequence number)

+	all routes to all destinations available at all times: less delay in route setup process

- Broken-link routing updates lead to heavy control O/H during high mobility (O/H proportional to number of nodes in network)

- Source must wait for table updates initiated by destination (potentially stale routing information)

Flooding: (reactive)

- Sender S broadcasts data packet P to all its neighbours

- Each node receiving P forwards P to its neighbours

- Sequence numbers used to avoid the possibility of forwarding the same packet more than once

- The destination D doesn’t forward the packet

- Potential for collisions, if a node receives a packet from 2 other neighbor nodes

+ simple

+ may be more efficient than other protocols when rate of information transmission is low enough that the overhead of explicit route discovery/maintenance incurred by other protocols is relatively higher (e.g. small data packets transmitted infrequently, many topology changes)

+ potentially higher reliability of data delivery (multi paths)

- potentially very high overhead (almost all nodes in the network receive the packet)

- potentially lower reliability of data delivery (simultaneously transmission to one node)

flooding of control packets to discover routes

Dynamic Source Routing (DSR): (reactive)

- When a node wants to send a packet, but doesn’t know the route, it initiates route discovery

- Source node S floods a Route Request(RREQ)

- Each node appends own identifier when forwarding RREQ

- Destination D on receiving the first RREQ sends a RREP (Route Reply) by reversing the route from RREQ

- Links have to be bi-directional

- Otherwise D needs a route discovery to S (piqqybacked RREP)

- “source routing” - the entire route is included in the packet header

Improvements: (reduce propagation of route requests, speed up route discovery)

- Each node caches a new route

- If a route is broken, take another route from its local cache

- Node X can send a RREP if it knows the route to node D

- BUT stale caches can reduce performance, invalid routes during some time and mobility, sender may try some routes before finding a good route

- route error (RERR) – if a connection J-D breaks J sends a RERR along the route back

- nodes hearing RERR update their route cache to remove the link J-D

pros:

- routes maintained only between nodes who need to communicate (reduces overhead)

- route caching can further reduce control overhead of route discovery

- single route discovery may yield many routes to the destination, due to intermediate nodes replying from local caches

- source routing quarantees loop-free routes

cons:

- packet header size grows with route length

- flood of RREQ may potentially reach all nodes in the network

- care must be taken to avoid collisions between route requests propagated by neighbouring nodes

- intermediate node may send RREP using a stale cached route, thus polluting other caches (solutions: static time outs, adaptive timeouts based on link stability)

Ad Hoc On-demand Distance Vector routing (AODV):

- RREQ forwarded similar to DSR

- When a node re-broadcasts a RREQ, it sets up a reverse path pointing towards the source

- The destination sends a RREP along the reverse path set up by RREQ forwarding

- Intermediate node may also send a RREP provided that it knows a more recent path than one previously known to sender S

- Destination sequence numbers used to determine whether the path known to an intermediate mode is more recent

- An intermediate node sends back an RREP if his DestSeqNum is bigger than the DestSeqNum from S  taking the route with the highest sequence number

- All neighbours are informed by route error messages, if a link breaks  update destination sequence numbers

- Avoid using old/broken routes, prevent formation of loops (count to infinity)

Pros:

- Routes established on-demand (reactive)

- Doesn’t use source routing, but maintains routing tables at each node, lower packet header O/H

- Destination sequence numbers used to find the latest route to destination

Cons:

- Multiple RREP in response to single RREQ can lead to heavy control O/H

- Periodic beaconing (“Hello”) leads to unnecessary bandwidth consumption

Flooding of control packets

How to reduce the scope of route request flood?

Location-Aided Routing (LAR):

- Exploits location info to limit scope of route request flood (e.g. using GPS)

- Only nodes within Request Zone (specified in the RREQ) forward route requests

- Each node must know its physical location

- If the destination is not in the request zone, the sender initiates another route discovery using a larger request zone

Variations:

- Adaptive Request Zone: Each node may modify the request zone included in the forwarded request, using more recent/accurate information

- Implicit Request Zone: node X forwards a route request received from Y, if node X is deemed to be closer to the expected zone as compared to Y

Pros:

- Reduces scope of route request floods

- Reduces overhead of route discovery

Cons:

- Nodes need to know their physical locations

- Doesn’t take into account possible existence of obstructions for radio transmissions

Distance Routing Effect Algorithm for Mobility (DREAM):

- Uses location and speed information, like LAR

- Uses flooding of data packets as the routing mechanism, unlike LAR

- Uses location information to limit the flood of data packets to a small region

Relative Distance Micro-Discovery Routing (RDMAR):

- Estimates distance between source and intended destination in number of hops

- Sender node sends RREQ with time-to-live (TTL) equal to the above estimate

- Hop distance estimate based on the physical distance that the nodes may have traveled since the previous route discovery, and transmission range

Geographic Distance Routing (GEDIR):

- Location of the destination node is assumed known

- Each node knows location of its neighbours

- Each node forwards packet to its neighbor closest to destination

Routing with guaranteed delivery:

- Improves on GEDIR, guarantees delivery (using location information) provided that a path exists from source to destination

- Routes around obstacles if necessary

Query localization:

- Flood without using physical information

- Route requests are propagated only along paths that are close to the previously known route

- Look for a new path that contains at most k new nodes that were not present in the previously known route

- Old route is piggybacked on a Route Request

+ reduces O7H of route discovery without using physical location information

+ can perform better in presence of obstructions by searching for new routes in the vicinity of old routes

- may yield routes longer than LAR (shortest route may contain more than k new nodes)

How to reduce redundant broadcasts

Broadcast storm problem:

- Collision if one node receives a packet simultaneously from multiple nodes

- Redundancy: a given node may receive the same route request from too many nodes, when one copy would have sufficed

Solutions:

- Probabilistic scheme: on receiving a route request for the first time, node will re-broadcast the request with probability p

- Re-broadcast with collision avoidance (wait a random delay, when channel is idle)

- Counter-based scheme: if node E hears more than k neighbors broadcasting a given route request, before it can itself forward it, then node E will not forward the request

- Distance-based scheme: if node E hears RREQ broadcasted by some node Z within physical distance d, then E will not re-broadcast the request

Link reversal algorithm:

- Link Failure  Any node (not destination) that has no outgoing links reverses all its incoming links

Partial reversal method:

- Reverses only those neighbours who have not themselves reversed links previously

Pros:

- Attempt to limit updated to routing tables at nodes in the vicinity of broken link

- Each node may potentially have multiple routes to a destination

Cons:

- Need a mechanism to detect link failure

- If network is partitioned, link reversals continue indefinitely

Temporally Ordered Routing Algorithm (TORA):

- Modifies the partial link reversal method

with detection of partitions

- When a partition is detected, all nodes in the partition

are informed and link reversals in that partition cease

- After node A has received the reflection from all its neighbours,

it knows that it is partitioned from destination D

- A sends a clear message (CLR)

- Each node maintains its one hop local topology info & has

Ability to detect partitions

- Directed acyclic graph is build

Pros:

- Detecting partitions and stopping non-productive link reversals

- Beneficial when many hosts want to communicate with a single destination

- Limits control packets for route configurations to a small region – less control O/H

Cons:

- Paths may not be shortest

- Concurrent detection of partitions & subsequent route deletion could result in temporary oscillations and transient loops

Design decision:

- When a link is repaired, it may not be assigned a direction (maybe route discovey)

- Reactive behavior possible

- Proactive behavior possible

- Decision should be made based on environment under consideration

Cluster-based routing

- Typically a leader is elected for each cluster of nodes

- Schemes may differ in: how clusters are determined, the way cluster head (leader) is chosen, duties assigned to the cluster head

Distance-vector routing: DSDV

- Each node maintains a routing table which stores

  - Next hop towards each destination

  - Cost metric for path to each destination

  - Destination sequence number created by destination itself

  - Sequence numbers used to avoid formation of loops

- Each node periodically forwards the routing table to its neighbours (increments and appends its sequence number when sending its local table)

- If the DestSeqNum from a packet send by a neighbor is bigger, than the one cached, then replace the route through that neighbor

Optimized Link State Routing (OLSR)

- Overhead of flooding link state information is reduced by requiring fewer nodes to forward the information

- Relies on selection of multipoint relays & calculates its routes to all known destinations through these nodes

- A broadcast from node X is only forwarded by its nultipoint relays

- Select the multipoint relay set among the one-hop neighbours, such that the set covers all two-hop neighbours

- Flood only through multipoint relays (multipoint relays are intermediate nodes)

- Reduces routing O/H & number of broadcasts  low connection setup time & reduced control O/H

Hybrid routing protocols

Zone Routing Protocol (ZRP):

- All nodes within hop distance of at most d from a node X are said to be in the routing zone of node X

- All nodes at hop distance exactly d are said to be peripheral nodes of node X’s routing zone

- Intra-zone routing: proactively maintain state information for links within a short distance from any given node

- Inter-zone routing: use a route discovery protocol for determining routes to far away nodes (similar to DSR with RREQ propagated via peripheral nodes)

Pros:

- Combines best features of proactive and reactive routing

- Reduces control O/H compared to RREQ flooding in on-demand approaches & periodic flooding of routing information packets on table-driven approaches

Cons:

- In absence of query control (ensuring redundant or duplicate RREQ not forwarded), produces higher control O/H (e.g. due to large overlapping of nodes routing zones

- Decision on zone radius significantly impacts performance

Landmark Ad hoc Routing (LANMAR) with group mobility:

- Landmark node is elected for a group of nodes that are likely to move together

- Scope is defined such that each node would typically be within the scope of its landmark node

- Each node propagates: link-state information corresponding only to nodes within its scope & distance-vector information for all landmark nodes

So far protocols discussed on a given network topology!

Controlling network topology by transmission power control

Power-aware routing:

- Define optimization criteria as a function of energy consumption, e.g.

  - Minimize energy consumed per packet

  - Minimize time to network partition due to energy depletion

  - Maximize node lifetime (duration before a node fails due to energy depletion),

- Assign a weight to each link

- Weight of a link may be a function of energy consumed when transmitting a packet on that link, as well as the residual energy level (low residual energy level may correspond to a high cost)

- Prefer a route with the smallest aggregate weight

- Possible modification to DSR to make it power aware:

  - RREQ’s aggregate the weights of all traversed links

  - Destination responds with a RREP to a RREQ if:

    - It is the first RREQ with a given (“current”) sequence number,

OR

    - Its weight is smaller than all other RREQs received with the current sequence number

MAC Layer for Wireless Networks

Wireless channel is a shared medium  need access control mechanism to avoid interference

Ideal MAC Algorithm:

- Efficient

- Fair (complicated to define/measure)

- Simple

Potentially fully decentralized

Hidden terminal problem

Solution Busy Tone:

- Receiver transmits busy tone when receiving data

- All nodes hearing busy tone keep silent

- Avoids interference from hidden terminals

- BUT requires a separate channel for busy tone

Solution MACA (Multiple Access Collision Avoidance):

- When node A wants to send a packet to node B, node A first sends a Request-to-Send (RTS) to B

- On receiving RTS, node B responds by sending Clear-to-Send (CTS), if node B is able to receive the packet

- When a node overhears a CTS, it keeps quiet for the duration of the transfer

Improve the reliability of wireless links:

- send an Acknowledgement (ACK) if a data packet is successfully received (e.g. in MACAW)

IEEE 802.11 DCF

- (optionally) uses RTS-CTS exchange to avoid hidden terminal problem

- Uses ACK to improve reliability

Collision avoidance

- CSMA/CA: wireless MAC protocols often use collision avoidance techniques, in conjunction with a carrier sense mechanism

- Carrier sense: when a node wishes to transmit a packet, it first waits until the channel is idle

- Collision avoidance: nodes hearing RTS or CTS stay silent; once channel is idle, the node waits (back off) for a randomly chosen duration before attempting to transmit

- when transmitting a packet, choose a backoff interval in the range [0,cw] (contention window)

- count down the backoff interval when medium is idle

- when backoff interval reaches 0, transmit RTS

- choosing a large cw leads to large backoff intervals and can result in larger overhead

- choosing a small cwleads to a larger number of collisions

- cw is choosen dynamically

Binary Exponentioal Backoff in DCF:

- When a node fails to receive CTS in response to its RTS, it increases the contention windod (cw is doubled)

- When a node successfully completes a data transfer, it restores cw to cw-min

- cw follows a sawtooth curve

Mild Algorithm in MACAW:

- Multiplicative increase, linear decrease

- MACAW: exponential increase, linear decrease

- Good when a large number of nodes contend for channel

Receive-initiated mechanism

- Receiver may send a Ready-to-Receive (RTR) message to sender

Fairness

Simplest definition: “all nodes should receive equal bandwidth”

- Unfairness occurs when one node has backed off much more than some other node

MACAW solution:

- When a node transmits a packet, it appends the cw value

- All nodes hearing that cw value use it for their future transmission attempts

- Cw is an indication of the level of congestion in the vicinity of a specific receiver node

- Using per-receiver cw is particularly useful in multi-hop environments, since congestion level at different receivers can be very different

Another MACAW proposal: RRTS

- When node A sends an RTS to B, while node C is receiving from D: node B cannot reply with CTS, sinse B knows that D is sending to C

- When transfer from C to D is complete, node B can send a Request-for-RTS(RRTS) to node A

- Node A may then immediately send RTS to node B

- Node B may not receive the RTS from A due to interference with transmission from C

Distributed Fair Scheduling (DFS):

- Fully distributed algorithm for achieving weighted fair queueing

- Choose backoff intervals proportional to (packet_size/weight) [assign a weight to each node]

- Trade-off between fairness and throughput

- Fairness in multi-hop case affected by hidden terminals

Balanced MAC:

- Variation on p-persistent protocol

- A link access probability p_ij is assigned tp each link (I,j) from node I to node J

- P_ij is a function of the 1-hop neighbours of node j and 1-hop neighbors of all neighbors of node i

- Node i picks a back-off interval, and when it counts to 0, node I transmits with probability p_ij

- Otherwise it picks another backoff interval, and repeats

- Fairness definition used here: max throughput/minthroughput

Reduction of energy consumption:

- Power save: turn off wireless interface when desirable

- Power control: reduce transmit power

Power Aware Multi-Access Protocol with Signalling (PAMAS)

- A node powers off its radio while a neighbor is transmitting to someone else

- What should node C do when it wakes up and finds that D is transmitting to someone else? (how long will the transfer last?)

- Uses a control channel separate from the data channel

- Node C on waking up performs a binary probe to determine the length of the longest remaining transfer

    - C sends a probe packet with parameter L

    - All nodes which will finish transfer in interval [L/2,L] respond

- Node C determines the duration of time to go back to sleep

- Disadvantages: use of a separate control channel; nodes have to be able to receive on the control channel while sending on the data channel; a node should be able to determine when probe responses from multiple senders collide

Another proposal:

- To avoid the probing, a node should switch off the interface for data channel, but not for the control channel (which carries RTS/CTS packets)

- + each sleeping node always knows how long to sleep by watching the control channel

- - this may not be useful when hardware is shared for the control and data channels

Pros power control: reduced interference & increased spatial reuse, energy saving

bad!

- Shorter hops typically preferred for energy consumption

- Plausible solution: RTS/CTS at highest power & DATA/ACK at lowest necessary power level

- BUT: A cannot sense C’s data transmission, and may transmit DATA to some other host this DATA will interfere at C

- Transmitting RTS at highest power level also reduces spatial reuse

Power control with IEEE 802.11 - Modification to avoid interference:

- RTS/CTS at highest power level, DATA/ACK at lowest required

- Increase DATA power periodically so distant hosts can sense transmission

Power Controlled Multiple Access (PCMA)

- If receiver node R can tolerate noise E, it sends a busy tone at power level C/E, where C is an appropriate constant

- When some node X receives a busy-tone at power level Pr, it may transmit at power level    Pt <= c/Pr

- + allows higher spatial reuse, as well as power saving using power control

- - need a separate channel for the busy tone; since multiple nodes may transmit the busy tones simultaneously, spatial reuse is less than optimal

Sender-based “autorate fallback” (ARF)

- Probing mechanisms

- Sender decreases bit rate after X consecutive transmission attempts fail

- Sender increases bit rate after Y consecutive transmission attempts succeed

Pros:

- Can be implemented at the sender, without making any changes to the 802.11 standard specification

Cons:

- Probing mechanism doesn’t accurately detect channel state

- Channel state detected more accurately at the receiver

- Performance can suffer

  - Sender will periodically try to send at a higher than optimal rate

  - When channel conditions improve, the rate is not increased immediately

Receiver-Based AutoRate MAC (RBAR):

- Sender sends RTS containing its best rate estimate

- Receiver chooses best rate for the conditions & sends it in the CTS (with help of SNR &

a-priori wireless channel model)

- Sender transmits DATA packet at new rate

- Information in DATA packet header implicitly updates nodes that heard old rate

Multi-Channel MAC: a simple approach

- Divide bandwidth into multiple channels

- Choose any one of the idle channels

- Use a single-channel protocol on the chosen channel (ALOHA, MACA)

Multi-channel MAC with “soft reservation”

- Similar to the simple scheme, but channel used recently for a successful transmission preferred

- Tends to “reserve” channels

- Requires all N channels to be continuously monitored

Another multi-channel MAC protocol:

- Use one (control) channel for RTS/CTS and remaining (data) channels for DATA/ACK

- Each host maintains NAV table, with one entry for each data channel

- Sender sends RTS to destination, specifying the channels that are free per sender’s table

- Receiver replies with CTS specifying a channel that it also thinks is free

- channel is used only if both sender and receiver conclude that it is free

Directional antennas on MAC and routing

Typically, Single-mode = omnidirectional

Here: directional antennas with multiple modes (beamforming in specified direction)

Vs.

Received Power is proportional to (Transmit power)*(TX Gain)*(RX Gain) (directional gain is higher)

Benefits:

- Reduced transmit power energy efficiency, reduced interference

- Increased PHY-layer security

- Reduced interference = increased capacity

- Increased spatial reuse  higher aggregate throughput

- Increased SINR higher throughput (Signal to Interference-plus-noise-ratio)

- Extended communication range  better network connectivity, fewer hop routes

Simple Tone Sense (STS) protocol:

- Based on busy tone signaling

- Each host is assigned a tone

- Tone frequency unique in each host’s neighbourhood

- When a host detects a packet destined to itself, it transmit a tone

- If a host receives a tone on directional antenna A, it assumes that some host in that direction is receiving a packet (cannot transmit using antenna A presently, OK to transmit using other antennas)

- Tone duration used to encode information (t1 implies transmitting node is busy, t2 implies the transmitting node successfully received a transmission from another node)

Directional MAC (D-MAC)

- IEEE802.11: node X is blocked, if node X received an RTS or CTS for ongoing transfer between two other nodes

- D-MAC: antenna T at node X is blocked, if antenna T received an RTS or CTS for ongoing transmission

- Transfer allowed using unblocked antennas

- Based on location information of the receiver, sender selects an appropriate directional antenna (assumes each node has physical location information of each neighbor via GPS)

D-MAC scheme 1: directional DATA&ACK, directional RTS (DRTS) and omnidirectional CTS (OCTS)

- Directional RTS including location information of node X

- Omnidirectional CTS including location information of nodes X and Y

- BUT collision-free ACK not guaranteed

D-MAC scheme 2

- Similar to scheme 1, but uses two types of RTS

- Directional DATA&ACK, omnidirectional CTS (OCTS)

- Omnidirectional RTS (ORTS) & directional RTS (DRTS)

  - If none of sender’s directional antennas blocked, send ORTS

  - Otherwise, when desired antenna not blocked, send DRTS

- pobability of ACK collision lower

- but fewer simultaneous transmissions

Performance comparison (1 vs. 2) depends on location, traffic, antenna:

D-Mac limitations: no guarantee of collision-free ACK, inaccurate/outdated location information can degrade performance

Conclusion:

- Pro: can allow more simultaneous transmissions by improving spatial reuse

- Cons: can increase ACK collisions

- Alternatives for determining location information should be considered

- Location information doesn’t always correlate well with direction

Basic DMAC protocol:

- Directional VCS with directional NAV (DNAV)

- Node listens omnidirectionally when idle

DATA & ACK transmitted and received directionally

- Directional RTS (DRTS) – received in omni mode

- Directional CTS (DCTS)

Directional NAV (DNAV)

- Nodes overhearing RTS or CTS set up directional NAV (DNAV)

for that Direction of Arrival (DoA)

- New transmission initiated only if direction of transmission

doesn’t overlap with DNAV

Pros:

- Better network connectivity

- Spatial reuse

Cons:

- Hidden terminals

- Deafness

- No DD links

Another DMAC variation:

- Assumptions:

  - Antenna model: several directional antennas which can all be used simultaneously

  - Omnidirectional reception is possible (by using all directional antennas together)

  - DoA can be determined when receiving omnidirectionally

  - Range of directional and omni transmissions are identical

- Sender sends omnidirectional RTS

- Receiver sends omnidirectional CTS

  - Receiver also records direction of sender by determining the antenna on which the RTS signal was received with highest power level

  - Similarly, the sender, on receiving CTS, records the direction of the receiver

- All nodes overhearing RTS/CTS defer transmissions

- Sender then sends DATA directionally to the receiver

- Receiver sends directional ACK

- Protocol takes advantage of reduction in interference due to directional transmission/reception of DATA

- All neighbors of sender/receiver defer transmission on receiving omnidirectional RTS/CTS

  - spatial reuse benefit not realized

Busy tone directional MAC (DBTMA):

- 3 channels: data channel, 2 busy tone channels (receive tone (BTr), transmit tone (BTt))

- Sender:

  - Sense BTr

  - If sensed busy, defer transmission

  - If BTr idle, transmit RTS to receiver

- Receiver:

  - On receiving RTS, sense BTt

  - If BTt idle, reply with CTS, and transmit BTr until DATA is completely received

- Sender:

  - On receiving CTS, transmit DATA and BTt both

- DBTMA reduces reduction in throughput caused by collisions by hidden terminals

- Directional antennas can be used to transmit the busy tone directionally

  - RTS/CTS, DATA, busy tones all may be sent directionally

  - Trade-offs similar to directional vs. omnidirectional transmission of RTS/CTS

Enhancing Basic DMAC – make use of DD links:

Multi-Hop RTS MAC (MMAC):

- Source routes RTS to D through adjacent DO neighbors (i.e. A-B-C-D)

When D receives RTS, it beamforms towards A, forming a DD link

- Reduces number of hops traversed by data

- Can improve delay and throughput

- Neighbor discovery overheads may offset the ads of MMAC

- Directional MAC protocols show improvements in total throughput and delay

- But not always

- Performance dependent on topology

Routing with directional antennas

Need to implement broadcast using directional transmissions to discover DD links

DSR over directional antennas:

Observations:

- Advantage of higher transmit range significant only at higher distance of separation

Grid distance = 200m – thus no gain with higher TX range of DDSR4 (350m) over IEEE802.11 (250m)

- However, DDSR4 has sweeping delay

- Thus route discovery delay is higher

Delayed RREP optimization:

- Due to sweeping – earliest RREQ need not have traversed shortest hop path

  - RREQ packets sent to different neighbors at different points of time

- If destination replies to first arriving RREP, it might miss shorter-path RREQ

- Optimize by having DSR destination wait before replying with RREP

Routing overhead:

- Using omni broadcast, nodes receive multiple (redundant) copies of same packet (broadcast storm problem)

Mobility:

- Link lifetime increases using directional antennas (higher transmission range – link failures are less frequent)

- Nodesmoving out of beam coverage in order of packet-transmission-time – low probability

- antenna handoff

  - If no response to RTS, MAC layer uses N adjacent antenna elements to transmit same packet

  - Route error avoided if communication re-established

DDSR performance: control overhead higher using DDSR, throughput of DDSR higher (even under mobility), latency in packet delivery lower using DDSR

Observations:

- Randomness in topology aids DDSR

- Voids in network topology bridged by higher transmission range (prevents partition)

- Higher transmission range increases link lifetime (reduces frequency of link failure)

- Antenna handoff due to nodes crossing antenna elements – not too serious

Other approaches for routing with directional antennas:

- Transmit RREQ in the last known direction of the receiver

- If the source S perceives receiver R to have been in direction d, then all nodes forward the route request from S in direction d

- + limits the forwarding of RREQ

- - effectively assumes that each node has a sense of orientation

- Directional antennas can improve routing performance, but suitable protocols adaptation necessary

- Directional antennas can be potentially beneficial, but also create difficulties in MAC & routing protocol desgn

Implementing Ad Hoc Networks over Wireless

Wireless Sensor Networks

WSNs can be seen as a special class or subset of ad hoc networks

Network of nodes that sense the environment & may control it

Enabling interaction between people & environment of interest

- M2M, D2D; IoT; smart home; smart cities

Motivation:

- Buildings self-detect structural faults

- Schools detect airborn toxins at low concentrations

- Cars monitor pollution, traffic jams, and weather

- Earthquake-rubbled building infiltrated with robots and sensors: locate survivors, evaluate structural damage

- Ecosystem infused with chemical, physical, acoustic, image sensors to track global change parameters

- Collaborative adaptive sensing of the atmosphere

Advantages of WSNs:

- Direct connection with its environment, often allowing measurements that have been impossible before

- No disturbance to measurement environment (e.g. animal or plant monitoring)

- Massive scale measurements possible

Applications:

- Habitat monitoring

- Building monitoring

- Environmental observation and forecasting systems

- Biomedical sensors

- Traffic monitoring

- Building automation

- Heavy machine monitoring & control

- Medical applications

Classification of WSNs:

- Sensor position

  - Static (habitat, biomedical)

  - Mobile (smart dust, biomedical)

- Goal-driven

  - Monitoring: real-time/non-real-time

  - Forecasting

- Communication medium

  - Radio frequency

  - Light

Low power consumption:

- Low power functional components

- Power-manageable components (deep-sleep, sleep, on states)

- Power management (power measurement, power budget allocation, control transitions between different power states)

Wireless communications:

- Communication mediums (radio frequency)

- Ad hoc versus infrastructure modes

- Topology

- Routing

Hardware is still a major issue (or constraint) for sensor networks

Low Energy Adaptive Clustering Hierarchy (LEACH)

Energy reduction due to local communication within cluster, data aggregation at CH

Useful network lifetime increased by distributing energy dissipation evenly through network

Radios for Ad Hoc Networks